JR301-SC-IW-SIP

Emergency Intercom

- 304 Stainless Steel body (SS316 available upon request)

- IP55-IP65 Waterproof rating

- Advanced noise reduction technology for HD calls

- Corrosion-resistant hardware

- CE, FCC, RoHS, IK10, IP65, ISO9001 standards

IP65

IK10

HD

# MAIN FEATURES

Housed in stainless steel with a wire-drawing finish and manufactured to high standards, these telephones offer enhanced resistance to vandalism and ensure reliable communication at all times.

Additionally, this series of phones has been tested in salt spray and is highly durable in humid and corrosive environments. Extremely easy to install, the cable entrance is located at the back of the phone to prevent artificial damage.

Ideal for parking lots, lifts/elevators, hotels, doorways, railway/metro platforms, hospitals, police stations, ATMs, stadiums, and outdoor building etc.

- Hands-free operation with a single vandal-resistant metal button for emergency calls

- Equipped with an LED light that blinks during incoming calls

- Noise-cancelling microphone

- Built-in 2W speaker

- Flush mounted design with tamper-proof screws

# TECHNICAL SPECIFICATIONS

| SIP Specifications |  |
| --- | --- |
| Power supply | PoE powered or 12V DC powered |
| Power adapter | 100~240VAC 50~60Hz / 12V DC |
| Power consumption | Idle 1W / Working 2.4W |
| SIP protocol | SIP 2.0 (RFC3261) |
| Network port parameters | WAN Port: 10/100Base- T RJ-45 for LAN, Auto MDIX，LAN Port: 10/100Base- T RJ-45 for PC, Auto MDIX |
| Voice coders | G.711 A/U, G.722 8000/16000, G.723, G.729 |
| Communication mode | Full duplex |
| Ringing volume | 90~95 dB (A) @ 1 meter |
| Technical Specifications |  |
| Compatible with most SIP-based IP-PBX servers that comply with SIP RFC 3261 |  |
| PoE 802.3af enabled (Power-over-Ethernet) or alternate power source |  |
| PoE power supply, supports connecting the external loudspeakers (No external power supply needed) |  |
| Supports automatic switching between primary and backup servers |  |
| Supports remote Telnet debugging |  |
| Supports two relays to remotely control external devices |  |
| Operating temperature -40˚C to 65˚C / Storage temperature -40°C to 75°C |  |
| M.T.B.F. exceeds 50,000 hours under normal operating conditions |  |
| Network Web Management |  |
| Fully configurable through any internet browser |  |
| Static IP address provisioning or DHCP support |  |
| Product self-diagnostic testing available through the web interface |  |
| Network-adjustable earpiece volume and microphone sensitivity |  |
| Firmware upload via web interface |  |
| Echo cancellation: Supports G.168, and hands-free can support 96ms |  |
| Supports SIP domain, SIP authentication (none, basic, MD5), and DNS name of server |  |
| DTMF: SIPINFO, INBAND, RFC2833 |  |
| Dials Plan: Flexible dial-up configuration, support hotline |  |
| Network Features |  |
| WAN/LAN: Supports Bridge mode |  |
| Supports static IP address |  |
| Supports DHCP to obtain an IP address on WAN port |  |
| Supports VLAN |  |
| Supports web config |  |
| Data Protocols: IPv4, TCP, UDP, ICMP, ARP, RTP, SRTP, ONVIF, SNMPv1, RTCP-XR, 802.1x, Static IP/DHCP IP Assignment,IEEE 802.1p/Q, QoS/ToS, HTTP/HTTPS/DHCP, NTP, FTP/TFTP, CDP/LLDP VLAN Configuration, LDAP |  |

Cat5e

Cat5e

APPLICATION

Cat5e

IP telephony system is an efficient and convenient communication solution that uses Internet connection to implement voice calls and other communication services. Compared with traditional telephone systems, IP telephone system has higher efficiency and conve- nience.

Server room

Control center

SIP Server

Cat5e

PoE Switch

Master station

Computer

JR301-SC-IW-SIP	JR301-SC-IW-SIP	JR301-SC-IW-SIP	JR301-SC-IW-SIP	JR301-SC-IW-SIP

OPTIONAL ACCESSORIES

Button

Multiple Options Available

JR-LP-02

LED Beacon Light

JR-HSA-10/20/25/30/35

Broadcast Horn

JR321-SC

Telephone Pillar

# APPLICATION SCENARIOS

Metro	Airport

Platform

Elevator

Clean room

Hospital

Bank

School

DIMENSIONS

| Package |  |
| --- | --- |
| Unit size | 216*146*40 (mm) |
| Shipping dimensions | 250*180*80 (mm) |
| Packaging materials | Foam padding, carton |
| Weight | Gross weight: 2 kg / Net weight: 1.5 kg |

Add: 4325 Woodland Park Dr. West Melbourne, Florida. 32904

Email: info@tuffcomm.com

Tel: +1 321-900-4599

Web: www.tuffcomm.com